L1

Part Number:	57060

UPC Number:	765809570603

Principal Application:	Various GM, Saturn, Saab (07-09)

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	3.430

Outer Diameter Top:	2.970

Outer Diameter Bottom:	Closed

Thread Size:	22X1.5 MM

By-Pass Valve Setting-PSI:	15

Anti-Drain Back Valve:	Yes

Beta Ratio:	2/75=10/25

Max Flow Rate:	10-12 GPM

Nominal Micron Rating:	21

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.750 	2.410 	0

L2

Part Number:	51431

UPC Number:	765809514317

Principal Application:	Iveco, Fiat-Allis, Hesston, Hitachi

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow/By-Pass

Media:	Paper

Height:	5.638

Outer Diameter Top:	4.240

Outer Diameter Bottom:	Closed

Thread Size:	3/4-16

By-Pass Valve Setting-PSI:	36

Anti-Drain Back Valve:	Yes

Burst Pressure-PSI:	400

Max Flow Rate:	7-9 GPM

Nominal Micron Rating:	29

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.809 	2.435 	0.080

L3

Part Number:	57117

UPC Number:	765809571174

Principal Application:	Mack, Renault Trucks - Centrifugal Element

All Applications

Style:	Cartridge Lube Metal Canister Filter

Service:	Lube

Type:	By-Pass

Height:	4.962

Outer Diameter Top:	3.880

Outer Diameter Bottom:	Closed

Inner Diameter Top:	0.396

Inner Diameter Bottom:	0.494

Beta Ratio:	2/8=5/9

Burst Pressure-PSI:	44

Max Flow Rate:	1-2 GPM

Nominal Micron Rating:	5

Gasket Diameters

Number	O.D.	I.D.	Thk.

Packed 	4.286 	4.058 	0.114

Packed 	4.142 	3.760 	0.240

L4

Part Number:	51522

UPC Number:	765809515222

Principal Application:	AMC (82-86), GM, Saturn (93-09), Jeep (81-86), Isuzu (03-09), Saab (06-09)

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	4.526

Outer Diameter Top:	2.921

Outer Diameter Bottom:	Closed

Thread Size:	13/16-16

By-Pass Valve Setting-PSI:	None

Anti-Drain Back Valve:	Yes

Burst Pressure-PSI:	290

Max Flow Rate:	9-11 GPM

Nominal Micron Rating:	15

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.734 	2.430 	0.226

L5

Part Number:	51675

UPC Number:	765809516755

Principal Application:	Mitsubishi Trucks

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow/By-Pass

Media:	Paper

Height:	5.981

Outer Diameter Top:	4.282

Outer Diameter Bottom:	Closed

Thread Size:	26X1.5 MM

By-Pass Valve Setting-PSI:	13-16

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.834 	2.462 	0.200

L6

Part Number:	33587

UPC Number:	765809335875

Principal Application:	Mack 12L E-Tech Engine (Secondary)

All Applications

Style:	Spin-On Fuel Filter

Service:	Fuel

Height:	10.365

Outer Diameter Top:	4.282

Outer Diameter Bottom:	Closed

Thread Size:	30X1.5 MM

By-Pass Valve Setting-PSI:	NONE

Max Flow Rate:	12-15

Nominal Micron Rating:	3

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	3.967 	3.587 	0.340

L7

Part Number:	51163

UPC Number:	765809511637

Principal Application:	Caterpillar Equipment (If High Efficency version required, use 51167 - if fire retardant version required, use 57720)

All Applications

Style:	Cartridge Lube Metal Canister Filter

Service:	Transmission

Type:	Full Flow

Media:	Paper

Height:	9.250

Outer Diameter:	4.000

Inner Diameter:	2.300

Beta Ratio:	2/20=10/24

Nominal Micron Rating:	23

Gasket Diameters

Number	O.D.	I.D.	Thk.

15163 8 	5.012 	4.734 	0.139

Housing Gasket Not Packaged with Filter .....Order Separately.

L8

Part Number:	51161

UPC Number:	765809511613

Principal Application:	Autocar, Bay City Shovel, Caterpillar, Gardner-Denver, Koehring, Komatsu

All Applications

Style:	Cartridge Lube Metal Canister Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	9.277

Outer Diameter:	3.968

Inner Diameter:	1.635

Beta Ratio:	2/20=17/33

Max Flow Rate:	18 GPM

Nominal Micron Rating:	25

Gasket Diameters

Number	O.D.	I.D.	Thk.

15449 	4.815 	4.585 	0.125

15286 	4.790 	4.630 	0.125

15108 8 	4.458 	4.250 	0.125

15460 8 	4.810 	4.580 	0.125

Housing Gasket Not Packaged with Filter .....Order Separately.

L9

Part Number:	51356

UPC Number:	765809513563

Principal Application:	Acura (02-09), Chrysler Imports (92-06), Ford Probe (93-97), Honda (01-09), Infiniti/Nissan (96-09), Mazda (71-00), Mercury Villager (99-00), Mitsubishi (90-09), Saturn (04-07), Outboard Marine Engines, Various HD Equip.

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	3.402

Outer Diameter Top:	2.685

Outer Diameter Bottom:	Closed

Thread Size:	20X1.5 MM

By-Pass Valve Setting-PSI:	8-11

Anti-Drain Back Valve:	Yes

Beta Ratio:	2/20=6/19

Burst Pressure-PSI:	363

Max Flow Rate:	9-11 GPM

Nominal Micron Rating:	19

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.475 	2.173 	0.233

L10

Part Number:	51040

UPC Number:	765809510401

Principal Application:	Daewoo (99-02), GM Cars/Trucks (76-09), Saab (94-98)

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	3.404

Outer Diameter Top:	2.921

Outer Diameter Bottom:	Closed

Thread Size:	18X1.5 MM

By-Pass Valve Setting-PSI:	None

Anti-Drain Back Valve:	Yes

Beta Ratio:	2/20=15/31

Burst Pressure-PSI:	325

Max Flow Rate:	7-9 GPM

Nominal Micron Rating:	19

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.734 	2.430 	0.226

L11

Part Number:	51282

UPC Number:	765809512825

Principal Application:	Isuzu Diesel Trucks (83-86) QD130 & C330 and Various Heavy Duty Equipment

All Applications

Style:	Cartridge Lube Metal Canister Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	6.500

Outer Diameter:	4.100

Inner Diameter Top:	0.570

Inner Diameter Bottom:	1.828

Ends:	Metal

Gasket Diameters

Number	O.D.	I.D.	Thk.

Packed (4)

L12

Part Number:	51133

UPC Number:	765809511330

Principal Application:	Adams, Allis-Chalmers, Case, Chevrolet Trucks (66-74), Detroit Diesel, Diamond T, Euclid, GMC, Gray Marine, Hercules, Hough, Joy, Letourneau-Westinghouse, Marion, M/F, Unit Crane, Worthington Compressors

All Applications

Style:	Cartridge Lube Metal Canister Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	9.000

Outer Diameter:	4.312

Inner Diameter:	1.450

Nominal Micron Rating:	25

Gasket Diameters

Number	O.D.	I.D.	Thk.

15119 	5.140 	4.590 	0.062

15127 	5.103 	4.787 	0.125

15333 8 	1.090 	0.765 	0.055

15402 8 	6.252 	5.852 	0.200

Housing Gasket Not Packaged with Filter .....Order Separately.

L13

Part Number:	51168

UPC Number:	765809511682

Principal Application:	Caterpillar, Clark, Hyster, LeRoi, Lorain, P&H, Other

All Applications

Style:	Cartridge Lube Metal Canister Filter

Service:	Lube/Transmission

Type:	Full Flow

Media:	Paper

Height:	8.937

Outer Diameter:	5.094

Inner Diameter:	3.375

Nominal Micron Rating:	25

Gasket Diameters

Number	O.D.	I.D.	Thk.

15193 8 	5.674 	5.468 	0.125

Housing Gasket Not Packaged with Filter .....Order Separately.

L14

Part Number:	51372

UPC Number:	765809513723

Principal Application:	Ford/Lincoln/Mercury (91-09), Mazda (00-09), Cadillac STS-V, XLR-V (06-09)

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	4.167

Outer Diameter Top:	3.660

Outer Diameter Bottom:	Closed

Thread Size:	22X1.5 MM

By-Pass Valve Setting-PSI:	16

Anti-Drain Back Valve:	Yes

Beta Ratio:	2/20=12/25

Burst Pressure-PSI:	275

Max Flow Rate:	11-13 GPM

Nominal Micron Rating:	19

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.834 	2.462 	0.200

L15

Part Number:	51334

UPC Number:	765809513341

Principal Application:	Acura (88-05), Chevrolet (85-88), Dodge (91-96), Ford (88-96), Honda (72-09), Hyundai (89-09), Isuzu (85-04), Kia (01-09), Mercury (87-91), Mitsubishi (89-98), Subaru (87-09) Atlas-Copco, Case, John Deere, Kobelco, Komatsu, Kubota, Onan, Toro & Yanmar Diesel, Other

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	3.194

Outer Diameter Top:	3.252

Outer Diameter Bottom:	Closed

Thread Size:	20X1.5 MM

By-Pass Valve Setting-PSI:	8-11

Anti-Drain Back Valve:	Yes

Beta Ratio:	2/20=22/40

Burst Pressure-PSI:	280

Max Flow Rate:	9-11 GPM

Nominal Micron Rating:	19

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.475 	2.173 	0.233

L16

Part Number:	51374

UPC Number:	765809513747

Principal Application:	BMW (65-91), Lancia (76-77) and M.G. (75-80), Various HD Equipment

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	3.529

Outer Diameter Top:	2.966

Outer Diameter Bottom:	Closed

Thread Size:	3/4-16

By-Pass Valve Setting-PSI:	8-11

Anti-Drain Back Valve:	Yes

Burst Pressure-PSI:	385

Max Flow Rate:	7-9 GPM

Nominal Micron Rating:	25

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.740 	2.370 	0.275

L17

Part Number:	51394

UPC Number:	765809513945

Principal Application:	GM (85-09), Daihatsu (88-92), Infiniti (91-96), Kawasaki Mules, Nissan (91-95), Toyota (80-09), Scion (04-09), Atlas-Copco, Briggs&Stratton, Cub Cadet, Generac, John Deere, Kubota, New Holland

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	2.977

Outer Diameter Top:	2.685

Outer Diameter Bottom:	Closed

Thread Size:	3/4-16

By-Pass Valve Setting-PSI:	8-11

Anti-Drain Back Valve:	Yes

Burst Pressure-PSI:	400

Max Flow Rate:	7-9 GPM

Nominal Micron Rating:	19

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.475 	2.173 	0.233

L18

Part Number:	51348

UPC Number:	765809513488

Principal Application:	Various Chrysler/Jeep (82-09), Various GM, Saturn (85-07), Lexus (90-09), Saab (67-09), Suzuki (86-02), Toyota (88-09), Yugo (86-89), Harley-Davidson, Various Lawn&Garden, Farm Equip.

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	3.404

Outer Diameter Top:	2.921

Outer Diameter Bottom:	Closed

Thread Size:	3/4-16

By-Pass Valve Setting-PSI:	8-11

Anti-Drain Back Valve:	Yes

Beta Ratio:	2/20=21/37

Burst Pressure-PSI:	275

Max Flow Rate:	7-9 GPM

Nominal Micron Rating:	19

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.734 	2.430 	0.226

L19

Part Number:	51085

UPC Number:	765809510852

Principal Application:	Chrysler/Dodge/Jeep Vehicles (91-08), Mitsubishi Eclipse (95-99); Caterpillar 3 Series Excavators, 9 Series Loaders, Kubota

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	3.790

Outer Diameter Top:	3.660

Outer Diameter Bottom:	Closed

Thread Size:	3/4-16

By-Pass Valve Setting-PSI:	8-11

Anti-Drain Back Valve:	Yes

Beta Ratio:	2/20=15/28

Burst Pressure-PSI:	290

Max Flow Rate:	7-9 GPM

Nominal Micron Rating:	20

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.834 	2.462 	0.200

L20

Part Number:	51607

UPC Number:	765809516076

Principal Application:	Cummins 5.9L "B" Series Eng, Dodge PU w/Diesel Eng. (89-02), Various HD Equip. (For High Efficiency Fluted Version Use 57620XE).

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	6.987

Outer Diameter Top:	3.670

Outer Diameter Bottom:	Closed

Thread Size:	1-16

By-Pass Valve Setting-PSI:	None

Beta Ratio:	2/20=17/27

Burst Pressure-PSI:	350

Max Flow Rate:	12-16 GPM

Nominal Micron Rating:	17

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.834 	2.462 	0.255

L21

Part Number:	51376

UPC Number:	765809513761

Principal Application:	Mitsubishi FK & FM Model Trucks

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	6.570

Outer Diameter Top:	4.282

Outer Diameter Bottom:	Closed

Thread Size:	1 1/8-16

By-Pass Valve Setting-PSI:	None

Burst Pressure-PSI:	420

Max Flow Rate:	18-20 GPM

Nominal Micron Rating:	28

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	3.967 	3.587 	0.340

L22

Part Number:	51515

UPC Number:	765809515154

Principal Application:	Chrysler Family of Cars/Trucks (57-70), Ford Family of Cars/Trucks (57-00), Mazda Trucks (94-00), Toyota Landcruiser (71-96) - (Two Quart version is 51773)

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	5.178

Outer Diameter:	3.660

Thread Size:	3/4-16

By-Pass Valve Setting-PSI:	8-11

Anti-Drain Back Valve:	Yes

Beta Ratio:	2/20=13/23

Burst Pressure-PSI:	290

Max Flow Rate:	7-9 GPM

Nominal Micron Rating:	19

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.834 	2.462 	0.200

L23

Part Number:	51602

UPC Number:	765809516021

Principal Application:	Agco, Case, Consolidated/Cummins 3.9L Engine, I-R, Komatsu, Other

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	5.214

Outer Diameter Top:	3.663

Outer Diameter Bottom:	Closed

Thread Size:	1-16

By-Pass Valve Setting-PSI:	None

Beta Ratio:	2/20=11/25

Burst Pressure-PSI:	375

Max Flow Rate:	12-16 GPM

Nominal Micron Rating:	17

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.834 	2.462 	0.255

L24

Part Number:	51036

UPC Number:	765809510364

Principal Application:	GM Cars + Trucks (80-05), Mercruiser, Volvo-Penta

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	4.828

Outer Diameter Top:	2.921

Outer Diameter Bottom:	Closed

Thread Size:	18X1.5 MM

By-Pass Valve Setting-PSI:	None

Anti-Drain Back Valve:	Yes

Beta Ratio:	2/20=12/26

Burst Pressure-PSI:	325

Max Flow Rate:	7-9 GPM

Nominal Micron Rating:	19

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.734 	2.430 	0.226

L25

Part Number:	51791

UPC Number:	765809517912

Principal Application:	Cat, Mack, Volvo (For High Efficiency 100% Glass Media use 51791XE - For Performance between 51791 + 51791XE, use 57791 uses cellulose/glass blend) - For Extended Drain Version Use 51791XD)

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	10.365

Outer Diameter Top:	4.282

Outer Diameter Bottom:	Closed

Thread Size:	1 1/8-16

By-Pass Valve Setting-PSI:	None

Beta Ratio:	2/20=11/25

Burst Pressure-PSI:	330

Max Flow Rate:	18-20 GPM

Nominal Micron Rating:	20

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	3.967 	3.587 	0.340

L26

Part Number:	51417

UPC Number:	765809514171

Principal Application:	Mack Trucks w/ Centri-Max Centrifugal By-Pass Lube System

All Applications

Style:	Cartridge Lube Metal Canister Filter

Service:	Lube

Type:	By-Pass

Height:	4.500

Outer Diameter Top:	3.830

Outer Diameter Bottom:	3.600

Inner Diameter Top:	0.556

Inner Diameter Bottom:	0.633

Gasket Diameters

Number	O.D.	I.D.	Thk.

Packed 	4.546 	4.156 	0.195

Packed 	1.762 	1.484 	0.139

L27

Part Number:	51754

UPC Number:	765809517547

Principal Application:	Ford Trucks 7.8L Diesel Eng. (87-93)

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	7.332

Outer Diameter Top:	4.282

Outer Diameter Bottom:	Closed

Thread Size:	1-12

By-Pass Valve Setting-PSI:	18-22

Burst Pressure-PSI:	435

Max Flow Rate:	12-16 GPM

Nominal Micron Rating:	16

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.834 	2.462 	0.200

L28

Part Number:	51810

UPC Number:	765809518100

Principal Application:	Equipment using Detroit Diesel 3-53, 4-53 Engines, Hino Trucks

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	6.219

Outer Diameter Top:	4.658

Outer Diameter Bottom:	Closed

Thread Size:	1 1/2-12

By-Pass Valve Setting-PSI:	None

Beta Ratio:	2/20=21/36

Burst Pressure-PSI:	335

Max Flow Rate:	28-30 GPM

Nominal Micron Rating:	30

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	4.320 	3.820 	0.365

L29

Part Number:	51268

UPC Number:	765809512689

Principal Application:	Caterpillar 3208 Engine, IHC Trucks, Allison Trans., Volvo Trucks

All Applications

Style:	Spin-On Lube Filter

Service:	Lube

Type:	Full Flow

Media:	Paper

Height:	5.218

Outer Diameter Top:	3.691

Outer Diameter Bottom:	Closed

Thread Size:	1 1/8-16

By-Pass Valve Setting-PSI:	None

Beta Ratio:	2/20=7/28

Burst Pressure-PSI:	560

Max Flow Rate:	18-20 GPM

Nominal Micron Rating:	25

Gasket Diameters

Number	O.D.	I.D.	Thk.

Attached 	2.729 	2.415 	0.255