CA4250P66K24T1A1EX

FAW CA4250P66K24T1E

FAW CA4250P66K24T1EX

Specifications and parameters of diesel engine

| No. | Item | CA6DM2-35E3 | CA6DM2-37E3 | CA6DM2-39E3 | CA6DM2-42E3 |
| --- | --- | --- | --- | --- | --- |
| 1 | Type | In-line, 4-stroke, water-cooling, 4-air-valve, turbocharged inter-cooled |  |  |  |
| 2 | Type of combustion chamber | Direct injection, ω type |  |  |  |
| 3 | Number of cylinder (mm) | 6 |  |  |  |
| 4 | Cylinder bore (mm) | 123 |  |  |  |
| 5 | Piston stroke (mm) | 155 |  |  |  |
| 6 | Compression ratio | 17.5 |  |  |  |
| 7 | Total piston displacement (L) | 11 |  |  |  |
| 8 | Rated power (kW) | 270 | 284 | 300 | 322 |
| 9 | Net power (kW) | 258 | 272 | 287 | 309 |
| 10 | Rated speed (r/min) | 1900 |  |  |  |
| 11 | Maximum torque (N.m) | 1550 | 1650 | 1750 | 1900 |
| 12 | Maximum torque speed (r/min) | 1100-1400 |  |  |  |
| 13 | Fuel supply system | Electric control high-pressure common rail |  |  |  |
| 14 | Exhaust contaminant | Meet GB 17691-2005 Stage III |  |  |  |
| 15 | Ignition order | 1-5-3-6-2-4 |  |  |  |
| 16 | Valve clearance (cold) (mm) | Inlet: 0.30 |  |  |  |
|  |  | Outlet: 3.0 |  |  |  |
| 17 | Minimum no-load stabilized speed (r/min) | 550±50 |  |  |  |
| 18 | Starting mode | Electric starting |  |  |  |
| 19 | Crankshaft rotation direction (flywheel-side-oriented) | Counterclockwise |  |  |  |
| 20 | Boundary dimensions (length× width×height) (mm) | 1365×773×1192 (without fan) |  |  |  |
| 21 | Net mass (kg) | 1050 |  |  |  |